# RAN Telemetry Parameters Available from FlexRIC/OAI

This document summarizes the MAC, RLC, PDCP, and GTP indication fields currently exposed through the FlexRIC Python xApp in the testbed, along with their present validation status.

# MAC

## Confirmed good to use

rnti - MAC-level UE identifier; matched the RLC RNTI in testing.

frame - NR system frame number; useful as radio context, not as the main wall-clock time source.

slot - NR slot index within the frame.

ul_mcs1 - Primary uplink modulation and coding scheme index.

dl_mcs1 - Primary downlink modulation and coding scheme index.

ul_bler - Uplink block error rate; populated correctly, including 0 under RFsim.

dl_bler - Downlink block error rate; populated correctly, including 0 under RFsim.

ul_sched_rb - Current number of scheduled uplink resource blocks.

dl_sched_rb - Current number of scheduled downlink resource blocks.

ul_aggr_prb - Cumulative uplink PRB usage.

dl_aggr_prb - Cumulative downlink PRB usage.

ul_aggr_retx_prb - Cumulative uplink retransmission PRBs.

dl_aggr_retx_prb - Cumulative downlink retransmission PRBs.

ul_curr_tbs - Current uplink transport block size.

dl_curr_tbs - Current downlink transport block size.

ul_aggr_tbs - Cumulative uplink transport block size; tracked RLC/PDCP uplink traffic closely.

dl_aggr_tbs - Cumulative downlink transport block size; tracked RLC/PDCP downlink traffic closely.

pusch_snr - Measured PUSCH signal-to-noise ratio.

pucch_snr - Measured PUCCH signal-to-noise ratio.

## Needs further investigation

ul_mcs2 - Available but not yet evaluated.

dl_mcs2 - Available but not yet evaluated.

ul_aggr_bytes_sdus - Observed as 0 during testing; population needs verification.

dl_aggr_bytes_sdus - Observed as 0 during testing; population needs verification.

ul_aggr_sdus - Available but not yet evaluated.

dl_aggr_sdus - Available but not yet evaluated.

bsr - Sometimes plausible, but occasionally wraps near 2^32; exact encoding/sentinel behavior must be verified.

phr - Populates and changes, but the exact encoding/units still need verification.

ul_num_harq - Available but not yet evaluated for usefulness.

dl_num_harq - Available but not yet evaluated for usefulness.

## Known not good / not useful in current implementation

wb_cqi - Consistently observed as 0; appears unpopulated.

ul_harq - Exposed as a raw SWIG object rather than directly usable scalar telemetry.

dl_harq - Exposed as a raw SWIG object rather than directly usable scalar telemetry.

# RLC

## Confirmed good to use

rnti - RLC UE identifier; matched the MAC RNTI in testing.

rbid - Radio bearer identifier.

mode - RLC mode identifier.

txbuf_occ_bytes - Transmit-buffer occupancy in bytes; became nonzero under downlink load.

txpdu_bytes - Cumulative transmitted RLC PDU bytes.

rxpdu_bytes - Cumulative received RLC PDU bytes.

txpdu_pkts - Cumulative transmitted RLC PDU packets.

rxpdu_pkts - Cumulative received RLC PDU packets.

txpdu_retx_bytes - Cumulative retransmitted RLC PDU bytes.

txpdu_retx_pkts - Cumulative retransmitted RLC PDU packets.

txsdu_bytes - Cumulative transmitted RLC SDU bytes.

rxsdu_bytes - Cumulative received RLC SDU bytes.

txsdu_pkts - Cumulative transmitted RLC SDU packets.

rxsdu_pkts - Cumulative received RLC SDU packets.

txsdu_wt_us - TX SDU head-of-line/waiting time in microseconds; became nonzero under downlink congestion.

## Needs further investigation

rxbuf_occ_bytes - Available but remained 0 in observed tests.

txpdu_dd_pkts - Available but behavior has not yet been characterized.

txpdu_dd_bytes - Available but behavior has not yet been characterized.

txpdu_segmented - Available but behavior has not yet been characterized.

txpdu_status_pkts - Available but behavior has not yet been characterized.

txpdu_status_bytes - Available but behavior has not yet been characterized.

rxpdu_dup_pkts - Available but behavior has not yet been characterized.

rxpdu_dup_bytes - Available but behavior has not yet been characterized.

rxpdu_dd_pkts - Available but behavior has not yet been characterized.

rxpdu_dd_bytes - Available but behavior has not yet been characterized.

rxpdu_ow_pkts - Available but behavior has not yet been characterized.

rxpdu_ow_bytes - Available but behavior has not yet been characterized.

rxpdu_status_pkts - Available but behavior has not yet been characterized.

rxpdu_status_bytes - Available but behavior has not yet been characterized.

rxsdu_dd_pkts - Available but behavior has not yet been characterized.

rxsdu_dd_bytes - Available but behavior has not yet been characterized.

## Known not good / not useful in current implementation

txbuf_occ_pkts - Remained 0 even when TX buffer occupancy in bytes was large.

rxbuf_occ_pkts - Remained 0 in current testing.

txsdu_avg_time_to_tx - Remained 0.0 in current testing.

txpdu_wt_ms - Remained 0 in current testing.

# PDCP

## Confirmed good to use

rbid - Radio bearer identifier.

mode - PDCP mode identifier.

txpdu_pkts - Cumulative transmitted PDCP PDU packets.

txpdu_bytes - Cumulative transmitted PDCP PDU bytes; matched downlink traffic trends across layers.

rxpdu_pkts - Cumulative received PDCP PDU packets.

rxpdu_bytes - Cumulative received PDCP PDU bytes; matched uplink traffic trends across layers.

txsdu_pkts - Cumulative transmitted PDCP SDU packets.

txsdu_bytes - Cumulative transmitted PDCP SDU bytes.

rxsdu_pkts - Cumulative received PDCP SDU packets.

rxsdu_bytes - Cumulative received PDCP SDU bytes.

rxpdu_oo_pkts - Out-of-order received PDCP packet counter.

rxpdu_dd_pkts - Discarded/duplicate-related PDCP packet counter.

txpdu_sn - PDCP transmit sequence number.

rxpdu_sn - PDCP receive sequence number.

## Needs further investigation

rxpdu_oo_bytes - Available but not yet meaningfully exercised.

rxpdu_dd_bytes - Available but not yet meaningfully exercised.

rxpdu_ro_count - Available but exact behavior has not yet been characterized.

rnti - Reported 1 while MAC/RLC reported the C-RNTI; likely a different internal identifier.

## Known not good / not useful in current implementation

None identified at this time.

# GTP

## Confirmed good to use

qfi - QoS Flow Identifier; populated during the active PDU session.

teid_gnb - gNB-side GTP tunnel endpoint identifier.

teid_upf - UPF-side GTP tunnel endpoint identifier.

## Needs further investigation

rnti - Reported 1 rather than the MAC/RLC C-RNTI; likely a different internal UE identifier.

## Known not good / not useful in current implementation

None identified at this time.