# OAI 5G MEC Video-VLM Demo

Technical Implementation Handoff | UE + MEC Server

| Purpose | Implementation guide for deploying and validating the live video-to-MEC VLM demo on the OAI testbed. |
| --- | --- |
| Code package | ue_mec_demo_complete.zip (ue/ + mec/) |

Demo in one sentence The UE container plays a prerecorded video, samples/crops/resizes it, compresses it as low-latency H.264 (I/P frames), sends it as RTP over UDP through the OAI 5G user plane to the MEC, while a separate WebSocket carries a natural-language question; the MEC decodes and buffers the received video, runs Qwen3-VL on a recent chronological frame window, and returns a structured semantic result to the UE.

# 1. Final System Architecture

UE Docker / OAI NR-UE MEC / DGX Spark prerecorded MP4 UDP :5000 ¦ ¦ v v sample FPS (1-30) -> crop/resize -> H.264 RTP jitter buffer ¦ I + P, B=0, GOP variable ¦ v v RTP/H.264 -> UDP :5000 -> OAI PDU session -> gNB -> UPF -> depay/decode ¦ ¦ +-> local post-H.264 preview +-> received preview ¦ Question/config -> WebSocket/TCP :8765 -------------------->+-> RGB ring buffer ¦ MEC VLM result <---------------------------------------------+-> Qwen3-VL

Media and control are intentionally separated. Video uses RTP/UDP for low-latency streaming; questions, configuration metadata, acknowledgements, and VLM results use reliable WebSocket/TCP.

The gNB does not terminate the application stream. The IP user-plane traffic traverses the OAI UE, NR air interface, gNB, core/UPF, and reaches the MEC application.

Both UE and MEC expose browser dashboards for real-time visualization without requiring X11/GUI support inside Docker.

# 2. Media and Control Protocols

| Path | Protocol / Port | Payload | Purpose |
| --- | --- | --- | --- |
| UE -> MEC video | RTP/H.264 over UDP, 5000 | H.264 Main profile, payload type 96 | Real-time compressed video |
| UE <-> MEC control | WebSocket over TCP, 8765, /ws/ue | JSON | Question/config to MEC; VLM result to UE |
| UE dashboard | HTTP, 8080 | HTML + MJPEG + JSON API | Transmitted-video preview and controls |
| MEC dashboard | HTTP, 8765 | HTML + MJPEG + JSON API | Received-video preview, RTP/VLM status |

## H.264 settings

H.264/x264 with I- and P-frames only; B-frames are disabled (bframes=0).

Default GOP/keyframe maximum: 10 frames (key-int-max=10), configurable at runtime. Because it is frame-based, GOP duration changes with sampled FPS.

tune=zerolatency and speed-preset=veryfast reduce encoder buffering/CPU cost for the live demo.

Default target bitrate: 400 kbit/s; default sampled resolution: 640x360; sampled FPS can be 1-30.

RTP MTU defaults to 1200 bytes to reduce IP-fragmentation risk through the tunneled 5G path.

rtph264pay config-interval=-1 repeats SPS/PPS with IDR frames for receiver recovery.

# 3. UE-Side Implementation

video.mp4 -> decodebin -> videorate -> crop -> resize -> I420 -> x264enc -> h264parse -> tee ¦ ¦ ¦ +-> H.264 decode -> JPEG -> UE browser preview +-> rtph264pay -> udpsink -> MEC_IP:5000

The prerecorded video resides inside the UE container (default /opt/ue-demo/video/demo.mp4).

udpsink sync=true is critical for file playback: it transmits according to media time instead of sending the entire file as fast as the CPU can process it.

The UE preview is decoded after H.264 encoding, so the displayed “transmitted video” reflects the actual codec/resolution/bitrate settings rather than the pristine source file.

Applying FPS, crop, resolution, bitrate, GOP, or RTP MTU changes rebuilds only the GStreamer media pipeline; the WebSocket control connection remains active.

At startup, the UE validates the route to the MEC and can fail fast unless it uses the expected OAI UE interface (default oaitun_ue1).

## UE control messages

UE -> MEC: ue_hello UE -> MEC: stream_config {fps, width, height, bitrate_kbps, gop_frames, b_frames=0, rtp_mtu, video_port} UE -> MEC: question {question_id, text, sent_unix_ns} MEC -> UE: vlm_result {question_id, event_detected, answer, confidence, details, inference_ms, ...}

# 4. MEC-Side Implementation

UDP :5000 -> rtpjitterbuffer -> rtph264depay -> h264parse -> avdec_h264 ¦ v decoded RGB frames ¦ +-----------------+-----------------+ ¦ ¦ v v JPEG MEC preview bounded ring buffer ¦ v sliding-window scheduler ¦ v Qwen3-VL-4B-Instruct ¦ v structured JSON ¦ WebSocket -> UE

The receiver uses a 50 ms RTP jitter buffer by default, H.264 depacketization/decoding, and bounded appsinks so a slow browser or VLM cannot back-pressure the media pipeline.

Decoded RGB frames are timestamped and inserted into a thread-safe rolling ring buffer. VLM inference is never run inside the GStreamer callback.

Default VLM schedule: every 2 seconds, inspect the previous 3 seconds, use at least 3 frames and uniformly select at most 10 frames across the temporal window.

Frames are resized to 448x252 for VLM inference only; the received stream and browser preview retain the UE-selected video resolution.

A new question clears the frame buffer by default, so it applies only to subsequently received video. If the question changes while inference is running, the stale result is discarded.

The VLM is Qwen/Qwen3-VL-4B-Instruct through Transformers. A mock backend is included for network/media validation before enabling GPU inference.

## VLM behavior

The model receives an ordered multi-image sequence with relative frame times, not a single still frame.

The prompt explicitly requires temporal evidence for actions such as taking, placing, removing, opening, entering, or leaving.

The model returns JSON fields: event_detected, answer, confidence, and details. Confidence is self-reported by the model and is not a calibrated probability.

Generation is deterministic by default (temperature=0.0) and limited to 128 new tokens.

# 5. Codebase Map

## UE directory

| File | Responsibility |
| --- | --- |
| ue/app.py | UE process entry point; loads configuration, performs route validation, starts FastAPI dashboard, GStreamer streamer, and WebSocket client. |
| ue/streamer.py | Builds and manages the prerecorded-video GStreamer pipeline; FPS sampling, crop/resize, x264 I/P encoding, RTP packetization/UDP transmission, post-encode preview, RTP byte/packet metrics, EOS/restart handling. |
| ue/control.py | Reliable UE-to-MEC WebSocket client with reconnect/keepalive; sends hello, stream configuration, questions, and receives MEC results. |
| ue/dashboard.py | FastAPI HTTP dashboard, MJPEG preview, status API, question submission, video start/stop/restart, and runtime video-configuration endpoints. |
| ue/models.py | Pydantic configuration models plus thread-safe shared state for stream, RTP metrics, active question, result, and preview. |
| ue/network.py | Runs ip route get for the MEC address and enforces the expected OAI UE tunnel/interface when strict checking is enabled. |
| ue/config.yaml | UE media, network, route-check, and dashboard parameters. |
| ue/requirements.txt | FastAPI/Uvicorn/WebSocket/YAML Python dependencies. |
| ue/install_deps.sh | Installs Python GI and GStreamer codec/RTP packages; creates a system-site-packages venv and validates required GStreamer elements. |
| ue/Dockerfile.example | Example for extending the existing OAI UE image while preserving the testbed-specific OAI startup behavior. |
| ue/README.md | UE deployment and operating notes. |

## MEC directory

| File | Responsibility |
| --- | --- |
| mec/app.py | MEC process entry point; FastAPI lifecycle, receiver, control hub, VLM engine, and inference scheduler integration. |
| mec/receiver.py | GStreamer UDP/RTP/H.264 receiver, jitter buffer, depacketizer/decoder, received-video preview, decoded RGB extraction, and RTP metrics. |
| mec/control.py | WebSocket server for UE hello/config/question messages, acknowledgements, active-question updates, and VLM-result delivery. |
| mec/models.py | MEC configuration models, decoded-frame dataclass/ring buffer, RTP statistics, and shared application state. |
| mec/scheduler.py | Periodic sliding-window inference logic; selects new temporal frames, runs VLM in a worker thread, rejects stale results, and formats vlm_result. |
| mec/vlm.py | Mock or real Qwen3-VL backend; preprocessing, chronological multi-image prompt, generation, JSON extraction/validation. |
| mec/dashboard.py | FastAPI received-video MJPEG dashboard, status API, manual infer-now and receiver-restart endpoints. |
| mec/config.yaml | RTP receiver, buffer, VLM, inference-window, and server parameters. |
| mec/check_environment.py | Checks Python modules, GStreamer elements, PyTorch/CUDA, and model runtime prerequisites. |
| mec/requirements.txt | Core FastAPI/Pydantic/NumPy/Pillow/YAML dependencies. |
| mec/requirements-vlm.txt | Transformers/Accelerate/Safetensors; intentionally does not overwrite the DGX Spark CUDA-enabled PyTorch environment. |
| mec/install_system_deps.sh | Installs GStreamer RTP/H.264 decoding and Python GI system packages. |
| mec/README.md | MEC deployment and model-enablement notes. |

# 6. Required Testbed Configuration

| Parameter | Default/example | Team action |
| --- | --- | --- |
| UE video source | /opt/ue-demo/video/demo.mp4 | Place/copy the prerecorded demo video inside the UE container. |
| MEC IP | 12.1.1.2 | Replace with the MEC address reachable through the OAI PDU session/UPF. |
| UE expected interface | oaitun_ue1 | Replace if the OAI UE tunnel interface has a different name. |
| Video UDP port | 5000 | Allow UE -> MEC UDP and ensure no conflicting service uses the port. |
| Control/dashboard TCP port | 8765 | Allow UE -> MEC WebSocket and browser -> MEC dashboard. |
| UE dashboard port | 8080 | Publish/map if the OAI UE container uses bridge networking; host networking needs no -p mapping. |
| VLM backend | transformers | Use mock first for integration, then switch to transformers. |
| DGX PyTorch/CUDA | testbed-specific | Use NVIDIA/DGX-Spark-compatible CUDA PyTorch; do not replace it with a generic CPU wheel. |

# 7. Deployment and Bring-Up Procedure

UE: copy/extend the OAI UE container with ue/, install the system/Python dependencies, place demo.mp4 at the configured path, and set MEC IP plus expected UE tunnel interface.

MEC: install GStreamer dependencies and core Python packages. Confirm UDP 5000 and TCP 8765 are reachable from the UE path.

MEC: set vlm.backend: mock and start python3 app.py --config config.yaml. Open http://MEC_IP:8765/.

UE: inside the container run ip route get MEC_IP. Confirm the route uses the OAI UE tunnel; with strict_route_check=true the app will refuse an incorrect route.

UE: start /opt/ue-demo-venv/bin/python app.py --config /opt/ue-demo/config.yaml and open http://UE_HOST:8080/.

Verify that the UE transmitted preview and MEC received preview are both live, and confirm RTP bitrate/packet counters increase.

Submit a question from the UE dashboard and verify the mock result returns over WebSocket.

Only after the full media/control path works, enable vlm.backend: transformers, validate the DGX Spark CUDA/PyTorch environment with mec/check_environment.py, then restart the MEC app and wait for the model status to become READY.

# 8. Testbed Validation Checklist

| Check | Expected result |
| --- | --- |
| UE route | ip route get MEC_IP shows dev oaitun_ue1 (or configured equivalent). |
| UE packet path | tcpdump on the UE tunnel shows UDP/5000 and TCP/8765. |
| MEC reception | tcpdump on MEC shows UDP/5000 and TCP/8765 from UE. |
| Video clocking | Prerecorded video plays in real time rather than transmitting at maximum file-read speed. |
| Codec | H.264 Main, B=0; GOP/bitrate/FPS changes are reflected after media-pipeline restart. |
| UE preview | Shows the post-H.264 local decode, not the original source video. |
| MEC preview | Shows the actual received/decoded stream in real time. |
| Control path | Question updates do not interrupt video; new question_id values are acknowledged/used. |
| Temporal inference | MEC uses multiple chronological frames; no single-frame-only action decision. |
| Stale result handling | Changing the question while inference is active does not display the old result as the new answer. |
| Resource isolation | VLM inference does not stall GStreamer frame reception. |
| Model phase | Mock end-to-end test passes before enabling Qwen3-VL. |

# 9. Recommended Initial Demo Parameters

| Component | Initial value | Rationale |
| --- | --- | --- |
| UE sampled FPS | 5 FPS | Enough temporal samples for simple human-object actions while keeping bitrate/compute manageable. |
| Resolution | 640x360 | Good starting point for bandwidth-constrained uplink and tabletop scenes. |
| H.264 target bitrate | 400 kbit/s | Initial value only; tune from measured stable OAI uplink goodput. |
| GOP | 10 frames | At 5 FPS, max keyframe distance is about 2 s. |
| RTP MTU | 1200 B | Conservative against tunnel/IP fragmentation. |
| MEC jitter buffer | 50 ms | Starting tradeoff between jitter tolerance and latency. |
| VLM window | 3 s | Captures before/during/after state transitions. |
| Inference interval | 2 s | Keeps model workload bounded while maintaining periodic semantic updates. |
| Max VLM frames | 10 | Uniformly covers the temporal window without processing every decoded frame. |
| VLM frame size | 448x252 | Reduces multimodal inference cost independently of network stream resolution. |
| Model | Qwen3-VL-4B-Instruct | Primary temporal multimodal model in the current implementation. |

# 10. Important Operational Notes

RTP bitrate shown by the UE counts RTP packet bytes before UDP/IP/GTP/NR overhead. Use packet capture/interface counters for end-to-end user-plane overhead measurements.

RTP sequence-gap packet loss on the MEC dashboard is an estimate, not a full PHY/RLC/PDCP loss measurement.

For formal measurements, prefer a sufficiently long prerecorded video with loop_video=false. Seek-based looping can create media timestamp discontinuities.

The WebSocket control client disables proxy use so private 5G traffic is not accidentally sent through host HTTP proxy settings.

The VLM result is semantic assistance, not a deterministic safety-critical decision. The current confidence field is model self-assessment.

Before tuning the model, first characterize stable UE-to-MEC goodput (for example with iperf3) and then tune H.264 bitrate/FPS/resolution against that measured link budget.

# 11. Quick Command Reference

# UE: verify 5G route and traffic ip route get <MEC_IP> tcpdump -ni <UE_TUN_IF> udp port 5000 tcpdump -ni <UE_TUN_IF> tcp port 8765 # UE: start application /opt/ue-demo-venv/bin/python /opt/ue-demo/app.py --config /opt/ue-demo/config.yaml # MEC: environment and traffic checks python3 check_environment.py tcpdump -ni any udp port 5000 tcpdump -ni any tcp port 8765 # MEC: start application python3 app.py --config config.yaml # Dashboards UE: http://<UE_HOST>:8080/ MEC: http://<MEC_IP>:8765/

Definition of done for the first integrated demo A prerecorded UE video is visibly transmitted through the OAI user plane as RTP/H.264; the same stream is visible after decoding at the MEC; a user can update a natural-language temporal question without stopping the stream; Qwen3-VL processes a recent multi-frame window at the MEC and returns a structured result that appears on the UE dashboard; packet/bitrate, buffer, inference, and connectivity status are visible for troubleshooting and demonstration.