# UE architecture requirements

Design decision record | Version 1.0 | October 26 2026

Aetheredge

Table of Contents

This record specifies the agreed UE behavior for the Aetheredge streaming MEC testbed. It complements MEC server architecture v2.1, MEC Decision Maker Architecture v4.4, and OAI/EdgeRIC architecture v1.5. U numbers identify the requirements in this record. The continuous cross-video behavior applies only to prerecorded video directories; camera streams retain their existing continuous-session behavior.

# 1. Platform and network identity

U01 Supported UE platforms. Support a Linux UE, including an NVIDIA Jetson AGX Orin, and an Android UE. Both clients shall use the same MEC control and media contracts where their platform APIs permit.

U02 Dynamic UE address. The UE shall discover and use the IP address assigned by the mobile core. A deployment shall not depend on a hard-coded address such as 12.1.1.2.

U03 Linux interface selection. The Linux client shall select the UE tunnel by interface name and bind media/control behavior to the address currently assigned to that interface. The expected default deployment interface is configurable, such as oaitun_ue1.

U04 Android network binding. The Android client shall bind its streaming session to the selected cellular network and update that binding if the network-assigned address changes.

# 2. Media sources and local storage

U05 Camera source. Both UE implementations may stream from a live camera. Camera streaming shall keep its existing single continuous-video lifecycle and shall not invoke prerecorded cross-video preparation or randomized selection.

U06 Single-file source. A UE may stream one explicitly selected prerecorded file using the existing one-video lifecycle. Selecting one file does not implicitly enable directory continuation.

U07 Prerecorded directory. A UE shall be able to store and select a local directory containing BDD100K or comparable prerecorded videos. Linux accepts a directory path. Android uses the system document-tree picker and retains read permission when the platform grants it.

U08 File discovery. Directory playback shall recursively discover the supported video files. The initial supported extensions are MP4, MOV, MKV, and M4V, matched without case sensitivity. An empty directory or unreadable selection shall fail clearly before streaming begins.

# 3. Randomized playlist behavior

U09 Random order. At the start of each playlist cycle, the UE shall shuffle the discovered prerecorded videos rather than use a fixed filename or upload order.

U10 Complete cycle. Every discovered video shall be selected exactly once per playlist cycle before any video is selected for the next cycle.

U11 Cycle boundary. When more than one video exists, the first selection of a new cycle shall not equal the final selection of the preceding cycle.

U12 Reproducibility. The UE shall support an optional shuffle seed for repeatable experiments. Without an explicit seed it shall choose a seed for the run. Logs shall record the seed, selected file, cycle number, and position.

# 4. Prerecorded-video handoff

U13 Advance preparation. Shortly before a prerecorded video reaches end of input, the UE shall request preparation of the already selected successor. The request shall identify the current video as the predecessor and provide an expected successor start time. The preparation lead is configurable by the MEC and defaults to five seconds.

U14 Separate session identity. The successor shall have a distinct video identifier, epoch, receiver/port assignment, frame sequence, configuration history, and per-video processing state. Frame numbering restarts for the new video.

U15 End notification. The UE shall send video_end for every completed prerecorded video so the MEC can close input, drain accepted work, preserve evidence, and reset state such as the object tracker.

U16 No end-of-video wait. After queuing the predecessor's video_end, the UE shall start the prepared successor locally without waiting for a MEC completion response. The first successor frame may immediately follow the last predecessor frame.

U17 Arrival-order tolerance. The MEC shall safely promote a prepared successor whether it observes the predecessor's end control message or the successor's first media descriptor first. A late terminal status for the predecessor shall not stop or replace the active successor.

U18 Late local startup. If local preparation finishes after the planned successor boundary, the UE shall begin from the current time. It shall not emit a catch-up burst for release times already in the past.

U19 Preparation failure. If advance preparation is rejected, times out, or fails, the UE shall retain the selected successor and request ordinary admission after the predecessor terminates. This fallback may introduce an inter-video delay but shall not silently skip the selection.

U20 Explicit stop. A user-requested end shall disable playlist continuation and end the active or prepared sessions cleanly.

# 5. Media and control consistency

U21 Frame types. A transmitted descriptor shall use type 1 for an IDR keyframe or type 3 for a P frame. The UE shall not transmit B frames or non-IDR I-frame descriptor types.

U22 Configuration scope. Application, SLO, input shape, and active bitrate/model configuration shall remain consistent across advance preparation unless a later architecture decision explicitly permits a per-video change. The successor receives fresh plan and configuration versions even when the selected values are inherited.

U23 Suspension scope. A prepared successor shall not inherit stale stop or suspension fields from its predecessor. Once active, it follows the ordinary versioned MEC transmission-control protocol.

U24 Evidence. UE and MEC logs shall retain playlist selection, preparation request and response, promotion trigger, fallback reason, video end, and final status with enough video and UE identity to reconstruct the handoff.

# 6. Acceptance checks

UA01 Address discovery. Start each UE with a core-assigned address that is not 12.1.1.2; verify that control and RTP traffic originate from the assigned UE interface/address.

UA02 Playlist ordering. With at least three files, verify that each file is selected once per cycle, two runs with the same seed have the same order, and the cycle boundary does not immediately repeat a file.

UA03 Camera isolation. Start a camera source and verify that it sends no successor-preparation request and retains the existing camera lifecycle.

UA04 Control-first handoff. Deliver video_end before the successor's first media packet; verify immediate promotion, a fresh tracker, and continued successor processing.

UA05 Media-first handoff. Deliver the successor's first descriptor before video_end; verify the same final state and verify that the later predecessor status cannot stop the successor.

UA06 Preparation failure. Reject or time out advance preparation and verify ordinary admission of the same selected file after the old session terminates.

UA07 Sustained playback. Run multiple complete playlist cycles and verify that session/receiver capacity is reclaimed, evidence remains attributable to the correct video, and memory and file descriptors remain bounded.

UA08 Platform validation. Run the checks on both the Linux UE and Android UE using their deployment toolchains. Android shall be compiled with JDK 17 and the configured Android SDK before installation.

# 7. Current implementation status

U01–U24 are implemented in the current working tree. Offline tests cover seeded playlist cycling and the two control/media arrival orders. Live Linux and Android execution, core-assigned address changes, sustained multi-cycle playback, and end-to-end MEC/RAN behavior remain acceptance work rather than completed validation.